Memory Hierarchy & Cache Performance

A Backend Engineer's Field Guide

CPU Registers → L1/L2/L3 Cache → RAM → SSD → Disk  |  Cache Lines  |  False Sharing  |  NUMA

Who this is for You are a junior backend engineer building Go services, REST APIs, and database-backed systems. This guide teaches you to reason about where data lives in hardware and write code that respects it. Every concept maps directly to HRIS payroll loops, LLM call center agents, and PostgreSQL on production servers.

# Part 1: The Memory Hierarchy

Every time your code reads a variable, the CPU must fetch it from somewhere. That 'somewhere' determines whether your instruction takes 0.3 nanoseconds or 10 milliseconds — a 30-million-to-one difference. Understanding where data lives is the foundation of all performance engineering.

## 1.1  Latency at Every Level

The hierarchy below is ordered from fastest and smallest to slowest and largest. The CPU always tries the fastest level first, falling back down until it finds the data.

| Storage Level | Latency | Typical Size | Location |
| --- | --- | --- | --- |
| CPU Registers | ~0.3 ns | ~1 KB | On the CPU die, inside the core |
| L1 Cache | ~1 ns | 32–64 KB per core | On-die, per core |
| L2 Cache | ~4 ns | 256 KB – 1 MB per core | On-die, per core |
| L3 Cache (LLC) | ~30–40 ns | 8–64 MB shared | On-die, shared across cores |
| RAM (DRAM) | ~100 ns | 16 GB – 1 TB | Off-chip DIMM |
| NVMe SSD | ~100,000 ns (0.1 ms) | 256 GB – 8 TB | PCIe attached |
| Spinning HDD | ~10,000,000 ns (10 ms) | 1–20 TB | Mechanical seek |

Mental model: the warehouse analogy Registers = your hands. Zero travel time. Holds 1-2 things at once. L1 cache = your desk. Grab anything in <1 second. Fits a few dozen pages. L2/L3 cache = your office bookshelf. Fast but requires getting up. RAM = the building's filing room. 100 steps away. Takes a minute. SSD = the warehouse across the street. Takes 5 minutes. HDD = the archives in another city. Half a day.

## 1.2  Why This Gap Exists

Speed and capacity trade off at the physics level. Fast memory (SRAM used in caches) requires 6 transistors per bit and consumes more power. Slow memory (DRAM used in RAM) uses 1 transistor + 1 capacitor per bit. You cannot have fast and large at the same time — the laws of physics prevent it.

Key implication: when you write a loop over 10 million records, the performance is not determined by your algorithm's O(n) complexity alone. It is determined by how many cache misses you generate. A cache-friendly O(n) algorithm will always beat a cache-hostile O(n) algorithm on real hardware.

# Part 2: Cache Lines & Cache Misses

## 2.1  The 64-Byte Rule

The CPU never fetches a single byte from RAM. It always fetches 64 bytes at a time, aligned to a 64-byte boundary. This 64-byte chunk is called a cache line.

Why 64 bytes? The CPU bets that if you needed byte X, you will probably need bytes X+1, X+2 ... soon. This is called spatial locality. The bet pays off when your data is laid out contiguously in memory.

Visualising a cache line fetch CPU needs: int64 at address 0x1040 (8 bytes) Actual fetch: addresses 0x1040 to 0x107F (64 bytes, the full cache line) Result: the neighbouring 7 int64s are now FREE — they are already in L1 Array loop: each cache miss gives you 8 int64s. 1M ints = ~125,000 misses. Linked list: each node pointer jumps to a random address. 1M nodes = ~1,000,000 misses.

## 2.2  Cache-Friendly vs Cache-Hostile Access Patterns

### Sequential access (cache-friendly)

// Go: iterating a slice — sequential, ~125k misses for 1M int64s sum := int64(0) for _, v := range salaries { // salaries []int64 sum += v }

### Random access (cache-hostile)

// Go: following a linked list — random jumps, ~1M misses for 1M nodes type Node struct { Value int64 Next *Node // pointer to a random heap address } sum := int64(0) for n := head; n != nil; n = n.Next { sum += n.Value // each access is likely a cache miss }

Real-world impact A linked-list traversal vs slice traversal of 1M int64s: ~4-8x slower. This is why every performance-sensitive Go service uses slices, not linked lists. It is also why PostgreSQL's heap scan (sequential) is faster than random index lookups for large row counts.

# Part 3: Cache-Friendly Struct Design in Go

## 3.1  Alignment Padding — The Silent Memory Tax

Go's compiler never reorders struct fields. It inserts padding bytes between fields to satisfy alignment requirements. Each type must start at an address that is a multiple of its size. If you order fields carelessly, you waste memory — and therefore cache lines.

### Alignment rules

| Type | Size | Alignment | Must start at address multiple of... |
| --- | --- | --- | --- |
| bool | 1 byte | 1 byte | Any address |
| int8 / uint8 | 1 byte | 1 byte | Any address |
| int16 / uint16 | 2 bytes | 2 bytes | Even addresses |
| int32 / uint32 / float32 | 4 bytes | 4 bytes | Multiple of 4 |
| int64 / uint64 / float64 | 8 bytes | 8 bytes | Multiple of 8 |
| pointer / string header | 8 bytes | 8 bytes | Multiple of 8 |

### Naive layout (wastes 14 bytes)

type Employee struct { Active bool // 1B @ offset 0 // [7B padding] ← wasted, aligning next int64 ID int64 // 8B @ offset 8 IsManager bool // 1B @ offset 16 // [7B padding] ← wasted, aligning next float64 Salary float64 // 8B @ offset 24 Level int32 // 4B @ offset 32 IsRemote bool // 1B @ offset 36 // [3B padding] ← wasted, aligning struct to 8B boundary } // Total: 40 bytes. Wasted: 17 bytes (43% overhead!)

### Optimised layout (zero wasted bytes)

type Employee struct { ID int64 // 8B @ offset 0 ← 8-byte types first Salary float64 // 8B @ offset 8 Level int32 // 4B @ offset 16 ← 4-byte types next Active bool // 1B @ offset 20 ← 1-byte types last, grouped IsManager bool // 1B @ offset 21 IsRemote bool // 1B @ offset 22 // [1B padding] ← minimal, just struct-end alignment } // Total: 24 bytes. Wasted: 1 byte. Saves 16 bytes per record. // At 1M records: saves 16 MB of RAM and cache pressure.

The rule Sort fields: largest alignment first, smallest last. Group booleans together at the end of the struct. Verify with: fmt.Println(unsafe.Sizeof(Employee{}))

## 3.2  Hot Path vs Cold Path — Only Load What You Touch

When a loop runs over millions of records and only touches 3 fields out of 20, you are dragging 17 cold fields into the CPU cache with every record fetch. The fix is to split your struct into a hot core and a cold profile.

### The problem — everything in one struct

// ❌ Bad: hot payroll loop drags 200B per record but uses only 16B type Employee struct { ID int64 // HOT: used in every loop IsActive bool // HOT: used in every loop DepartmentID int32 // HOT: used in every loop // ---- cold below, never touched in payroll loop ---- FullName string // 16B (header + heap pointer) HireDate time.Time // 24B Address string AvatarURL string Bio string } // 200 bytes per record. Cache line holds only 0.3 records. // 1M records = ~3.1M cache line fetches

### The fix — split hot and cold

// ✅ Good: separate hot core from cold profile type EmployeeCore struct { ID int64 // 8B DepartmentID int32 // 4B IsActive bool // 1B _ [3]byte // explicit pad to 16B total } // 16 bytes per record. Cache line holds 4 records. // 1M records = ~250,000 cache line fetches — 12x fewer misses type EmployeeProfile struct { EmployeeID int64 // foreign key back to core FullName string HireDate time.Time Address string AvatarURL string Bio string } // Load only when rendering a profile page — never in hot loops

### The database equivalent — apply the same thinking in SQL

-- ❌ Bad: loads entire 400B row into 8KB Postgres buffer page SELECT * FROM employees WHERE is_active = true; -- ✅ Good: projects only hot columns, index-only scan possible SELECT id, department_id, is_active FROM employees WHERE is_active = true; -- ✅ Even better: split cold columns to a separate table -- employees: id, department_id, is_active, salary, level -- employee_profiles: employee_id, full_name, hire_date, address, avatar_url, bio -- Index covering hot columns (index-only scan for attendance job) CREATE INDEX idx_employees_hot ON employees (is_active, department_id) INCLUDE (id);

## 3.3  Array of Structs vs Struct of Arrays (AoS vs SoA)

When processing large batches of the same type of data — like a payroll run — the layout of your data in memory determines cache efficiency for column-wise operations.

### Array of Structs (AoS) — intuitive but column-hostile

// AoS: records[0].Salary, records[1].Salary are 64 bytes apart type PayrollRecord struct { EmployeeID int64 // 8B BaseSalary float64 // 8B Bonus float64 // 8B TaxRate float32 // 4B // total: 28B per record } var records []PayrollRecord // Summing all salaries: touches every 28B record to get 8B salary // 64-byte cache line contains ~2 records — 20B of each is wasted total := 0.0 for _, r := range records { total += r.BaseSalary }

### Struct of Arrays (SoA) — column-friendly for batch ops

// SoA: all salaries contiguous — like a database column store type PayrollBatch struct { EmployeeIDs []int64 // contiguous column BaseSalaries []float64 // contiguous column — 8 per cache line Bonuses []float64 TaxRates []float32 } // Summing all salaries: walks BaseSalaries only // 64-byte cache line = 8 float64s — zero wasted bytes total := 0.0 for _, s := range batch.BaseSalaries { total += s } // This is why columnar databases (ClickHouse, Redshift, Parquet) // are faster than row-oriented ones for analytics queries.

# Part 4: False Sharing in Multi-Threaded Code

## 4.1  What Is False Sharing

False sharing occurs when two goroutines (or threads) update different variables that happen to reside on the same 64-byte cache line. Even though the variables are logically independent, the CPU cache coherence protocol treats them as shared data — forcing expensive cache invalidations on every write.

The symptom: your goroutines show no lock contention, CPU utilisation is healthy, but throughput is 3-10x lower than a single-threaded version. The perf trace looks clean — the problem is invisible to standard profiling.

## 4.2  The MESI Protocol — Why It Hurts

Modern multi-core CPUs maintain cache coherence using the MESI protocol (Modified, Exclusive, Shared, Invalid). When Core 0 writes to a cache line, it broadcasts an 'invalidate' message to all other cores. Core 1 must then re-fetch the entire 64-byte cache line from L3 or RAM before it can write — even if Core 1 is only touching a different field within that line.

False sharing step by step 1. Core 0 writes counterA. Broadcasts: 'invalidate line 0x2000 on all cores' 2. Core 1 wants to write counterB (same line). Its copy is now Invalid. 3. Core 1 must fetch the line from L3/RAM (~30-100 ns stall). 4. Core 1 writes counterB. Broadcasts: 'invalidate line 0x2000 on all cores'. 5. Core 0 wants to write counterA again. Its copy is now Invalid. Repeat. Result: two goroutines serialise on a cache line even with no mutex.

## 4.3  Detecting and Fixing False Sharing

### The bad code

// ❌ Bad: both counters share a 64-byte cache line type Metrics struct { RequestsHandled int64 // offset 0 ErrorsLogged int64 // offset 8 ← same cache line as above! } var m Metrics // Worker 1 increments m.RequestsHandled // Worker 2 increments m.ErrorsLogged // They ping-pong the cache line between cores — false sharing

### The fix — padding to 64 bytes

// ✅ Good: each counter on its own cache line type PaddedInt64 struct { Value int64 _ [56]byte // pad to 64 bytes total (8 + 56 = 64) } type Metrics struct { RequestsHandled PaddedInt64 ErrorsLogged PaddedInt64 } // Now each field is on a different cache line. // Core 0 writing RequestsHandled does NOT invalidate ErrorsLogged. // Benchmark result: ~3.5x throughput improvement on 2 goroutines.

### Benchmark — run this yourself

package main import ( "fmt" "runtime" "sync" "time" ) const N = 500_000_000 type Shared struct{ A, B int64 } type Padded struct{ V int64; _ [56]byte } func runShared() time.Duration { var c Shared var wg sync.WaitGroup wg.Add(2) t := time.Now() go func() { defer wg.Done(); for i := 0; i < N; i++ { c.A++ } }() go func() { defer wg.Done(); for i := 0; i < N; i++ { c.B++ } }() wg.Wait() return time.Since(t) } func runPadded() time.Duration { var c [2]Padded var wg sync.WaitGroup wg.Add(2) t := time.Now() go func() { defer wg.Done(); for i := 0; i < N; i++ { c[0].V++ } }() go func() { defer wg.Done(); for i := 0; i < N; i++ { c[1].V++ } }() wg.Wait() return time.Since(t) } func main() { runtime.GOMAXPROCS(2) fmt.Printf("False sharing: %v\n", runShared()) fmt.Printf("Padded: %v\n", runPadded()) } // Expected output on a modern machine: // False sharing: 3.1s // Padded: 0.9s

Where you will hit this in the HRIS / call center agent LLM agent: each worker goroutine tracks its own request counter in a shared metrics struct. Attendance job: multiple goroutines update per-department aggregates in a map or slice. API server: per-worker error counters or rate-limit token buckets in a shared struct. Fix: always pad hot per-goroutine counters to 64 bytes when concurrently written.

# Part 5: NUMA Architecture

## 5.1  What Is NUMA

On multi-socket servers (2, 4, or 8 CPU sockets), RAM is not equally accessible to all cores. Each socket has a local memory bank physically wired to it. Accessing local RAM takes ~100 ns. Accessing the other socket's RAM crosses the QPI/UPI interconnect and takes ~200-400 ns — a 2-4x penalty.

NUMA stands for Non-Uniform Memory Access. It is the default architecture on any AWS instance with 32+ vCPUs, all bare-metal servers, and most production database servers you will ever run.

## 5.2  Topology of a 2-Socket Server

| NUMA Node 0 (Socket 0) Cores 0-15 L3 Cache: 32 MB Local RAM: 64 GB Local access: ~100 ns | NUMA Node 1 (Socket 1) Cores 16-31 L3 Cache: 32 MB Local RAM: 64 GB Cross-node access: ~250 ns |
| --- | --- |

The two sockets are connected by Intel's UPI (Ultra Path Interconnect) or AMD's Infinity Fabric. The node distance is reported as a ratio: local = 10, remote = 21 means remote access costs 2.1x more.

## 5.3  Why Databases Suffer Most

PostgreSQL allocates a large shared_buffers pool at startup. If the OS allocates this pool across both NUMA nodes, every buffer read might cross the interconnect — and Postgres reads its buffer pool millions of times per second. The result is a server with more RAM and more cores running slower than the old single-socket machine it replaced.

## 5.4  Diagnosing NUMA Problems

# 1. Check if the server has multiple NUMA nodes numactl --hardware # 2. Check if Postgres is suffering cross-NUMA allocation numastat -p $(pgrep -f 'postgres -D' ¦ head -1) # Look for: numa_miss >> 0 and other_node >> 0 # 3. Check memory per node numastat

## 5.5  Fixing NUMA for PostgreSQL

# Option 1: Pin Postgres to node 0 (CPU + memory) numactl --cpunodebind=0 --membind=0 /usr/bin/postgres -D /var/lib/postgresql/data # Option 2: Add to systemd service file # /etc/systemd/system/postgresql.service ExecStart=numactl --cpunodebind=0 --membind=0 /usr/bin/postgres $PGARGS # Option 3: Interleave memory across both nodes # (worse latency but avoids hot-node exhaustion on very large servers) numactl --interleave=all /usr/bin/postgres -D /var/lib/postgresql/data # postgresql.conf — keep shared_buffers within one node's RAM # On 2-socket 128GB server: shared_buffers = 48GB # fits comfortably in 64GB node 0 huge_pages = on # reduces TLB pressure on large NUMA pools

Rule of thumb for HRIS production servers If your RDS or EC2 instance has >= 32 vCPUs, assume it has 2 NUMA nodes. If you self-host PostgreSQL on a bare-metal server, always run numactl --hardware first. Keep shared_buffers to 70-75% of one node's RAM, not total system RAM. A single numactl command can give 20-30% throughput improvement with zero code changes.

# Part 6: 20 Practice Questions

These questions test understanding, not memorisation. For each one, reason through the answer from first principles — do not look up the answer first.

| # | Question |
| --- | --- |
| 1 | If a CPU needs a value at memory address 0x1000, what range of addresses gets loaded into cache? |
| 2 | An array of 1,000,000 int64 values is iterated sequentially. Approximately how many cache line misses occur? |
| 3 | A linked list of 1,000,000 nodes is traversed. Why is the cache miss count potentially much higher than the array case? |
| 4 | You have a struct with fields: bool (1B), int64 (8B), bool (1B), float64 (8B). What is the size of this struct due to padding? |
| 5 | Rewrite the struct from Q4 in the optimal field order. What is the new size? |
| 6 | Two goroutines each write to their own int64 counter 500 million times. They run 10× slower than expected. What is the likely cause? |
| 7 | What is the minimum struct size that guarantees a field lives on its own cache line? Why? |
| 8 | Explain in one sentence what MESI stands for and why it matters for multi-core performance. |
| 9 | Your HRIS payroll job sums all employee salaries. You have a []PayrollRecord where each record is 64 bytes but salary is only 8 bytes. What percentage of each cache line fetch is wasted? |
| 10 | How would you restructure the data in Q9 to eliminate this waste? What is the trade-off? |
| 11 | A server has 2 NUMA nodes. Core 0 (node 0) allocates a 32 GB buffer. Core 20 (node 1) reads from it. What is the approximate latency penalty vs. local access? |
| 12 | What command tells you whether your PostgreSQL process is suffering from cross-NUMA memory access? |
| 13 | Your Go service has a hot struct accessed by 8 goroutines concurrently, with fields updated independently. What is the risk, and how do you fix it? |
| 14 | When does a cache miss become a cache eviction? What determines which cache line gets evicted? |
| 15 | In a 2-socket NUMA machine with 128 GB RAM total, what is the maximum safe value for PostgreSQL shared_buffers to avoid cross-NUMA allocation? |
| 16 | Explain why SELECT * FROM employees is cache-unfriendly at the PostgreSQL buffer pool level when you only need 3 columns. |
| 17 | A database table has columns: id (8B), name (varchar), is_active (1B), salary (8B), bio (text), avatar_url (varchar). Which columns should be in the main table vs. a split profile table for a hot attendance query? |
| 18 | Write the numactl command to pin a PostgreSQL process to NUMA node 0 for both CPU and memory. |
| 19 | Your Go struct is 72 bytes. How many cache lines does one instance span? Does that matter? When does it start to matter? |
| 20 | Describe a real scenario in an HRIS or call center agent system where false sharing could silently destroy throughput. |

# Part 7: Mini Project — HRIS Memory Profiler

Build this Go program. It benchmarks the concepts from this guide against each other using data that mirrors your real HRIS workload. Run each benchmark, record the numbers, and explain why each result is what it is.

## 7.1  Project Structure

hris-bench/ ├── main.go ├── structs.go // struct definitions (bad and good layouts) ├── bench_layout.go // benchmark struct size and cache pressure ├── bench_sharing.go // benchmark false sharing vs padded └── bench_access.go // benchmark AoS vs SoA for payroll sum

## 7.2  structs.go — Define Your Types

package main import "time" // --- NAIVE LAYOUT (bad) --- type EmployeeBad struct { Active bool ID int64 IsManager bool Salary float64 DeptID int32 HireDate time.Time IsRemote bool FullName string } // --- OPTIMISED LAYOUT (good) --- type EmployeeGood struct { ID int64 Salary float64 HireDate time.Time FullName string DeptID int32 Active bool IsManager bool IsRemote bool _ [1]byte } // --- HOT CORE only --- type EmployeeCore struct { ID int64 DeptID int32 Active bool _ [3]byte } // --- AoS (bad for column sums) --- type PayrollRecord struct { EmployeeID int64 Salary float64 Bonus float64 TaxRate float32 } // --- SoA (good for column sums) --- type PayrollBatch struct { EmployeeIDs []int64 Salaries []float64 Bonuses []float64 TaxRates []float32 } // --- False sharing (bad) --- type WorkerMetricsBad struct { Handled int64 Errors int64 } // --- Padded (good) --- type WorkerMetricsGood struct { Handled int64 _ [56]byte Errors int64 _ [56]byte }

## 7.3  main.go — Print Sizes and Run Benchmarks

package main import ( "fmt" "unsafe" ) func main() { // Task 1: Print struct sizes and spot the difference fmt.Println("=== Struct Sizes ===") fmt.Printf("EmployeeBad: %d bytes\n", unsafe.Sizeof(EmployeeBad{})) fmt.Printf("EmployeeGood: %d bytes\n", unsafe.Sizeof(EmployeeGood{})) fmt.Printf("EmployeeCore: %d bytes\n", unsafe.Sizeof(EmployeeCore{})) fmt.Printf("PayrollRecord: %d bytes\n", unsafe.Sizeof(PayrollRecord{})) fmt.Println() // Task 2: Run the benchmarks (implement in bench_*.go) fmt.Println("=== AoS vs SoA (payroll sum) ===") benchAoS() benchSoA() fmt.Println() fmt.Println("=== False Sharing ===") benchSharedCounters() benchPaddedCounters() }

## 7.4  Your Tasks

- Implement bench_access.go: fill 1,000,000 PayrollRecords and a PayrollBatch with random salaries. Benchmark summing all salaries in each. Print elapsed time for both.

- Implement bench_sharing.go: two goroutines each increment their counter 500M times using the bad struct, then the padded struct. Print elapsed time for both.

- Run the program. Record all 4 numbers. For each pair, calculate the speedup ratio.

- Add a third benchmark: hot path vs full struct. Fill 1M EmployeeBad and 1M EmployeeCore. Sum the DeptID field in both. Which is faster? Why?

- Write a comment in the code explaining the result of each benchmark in your own words.

Expected results (reference — try before peeking) EmployeeBad size: ~80 bytes. EmployeeGood: ~56 bytes. EmployeeCore: 16 bytes. AoS salary sum: ~3-5ms. SoA salary sum: ~1-2ms. (~3x faster) False sharing: ~3s. Padded: ~0.9s. (~3.5x faster) EmployeeCore hot loop: ~2x faster than EmployeeBad hot loop for same sum operation.

# Quick Reference Card

| Concept | One-line rule |
| --- | --- |
| Cache line size | Always 64 bytes — the minimum unit the CPU fetches from RAM |
| Cache miss cost | L1 hit = 1 ns. RAM miss = 100 ns. 100x penalty. |
| Struct layout | Largest fields first. Group bools at end. Verify with unsafe.Sizeof. |
| Hot/cold split | Separate frequently-accessed fields from rarely-touched ones. |
| AoS vs SoA | Column operations? Use SoA. Random record access? AoS is fine. |
| False sharing | Pad hot per-goroutine counters to 64 bytes each. |
| NUMA detection | numactl --hardware — if 2+ nodes, NUMA matters. |
| NUMA fix | numactl --cpunodebind=0 --membind=0 postgres |
| DB hot/cold | SELECT only the columns you need. Split cold columns to a separate table. |
| Measure first | Always benchmark before and after. Use go test -bench. |